# Project Report: Automatic Plant Watering System (Arduino)

## 1. Project Goal

The goal of this project is to create a simple automatic plant watering system that measures soil moisture and turns on a water pump when needed.

The system should:

- Show soil moisture on an LCD display.

- Automatically turn on a pump via a relay when moisture is low.

- Be safe and easy to assemble.

- 2. Project Components

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino Uno | 1 | Microcontroller controlling all devices |
| I²C LCD 16x2 | 1 | Display showing moisture level |
| Soil Moisture Sensor | 1 | Measures soil moisture in percentage |
| 5V Relay | 1 | Switches pump power |
| 5V Water Pump | 1 | Waters the plants |
| Wires | – | For connecting all parts |
| 5V Power Supply | 1 | For the pump (can be separate) |

3. Wiring Diagram

Text summary:

- LCD: VCC → 5V, GND → GND, SDA → A4, SCL → A5

- Sensor: VCC → 5V, GND → GND, OUT → A0

- Relay: VCC → 5V, GND → GND, IN → D8

- Pump: +5V → through relay → pump, - → GND (external supply)

- Common GND: Arduino + power supply + relay + sensor

4. How the System Works

- Arduino reads the moisture sensor every second.

- The raw value is converted into a percentage.

- If moisture drops below a set threshold (e.g., 30%), Arduino turns on the relay, powering the pump.

- The pump runs until moisture reaches the upper threshold (e.g., 40%) or until a maximum watering time (e.g., 30 seconds) is reached.

- Moisture levels and raw data are shown on the LCD.

- Data is also sent to the Serial Monitor for debugging.

5. Component Specifications

- Arduino Uno: 5V, up to 40 mA per pin, USB max 500 mA

- I²C LCD 16x2: 5V, ~20 mA

- Soil Moisture Sensor: Analog, 0–1023, capacitive recommended for durability

- 5V Relay: Coil ~70–100 mA, active LOW

- 5V Water Pump: Current depends on model (0.2–1 A for mini pumps)

- 5V Power Supply: Must provide current ≥ pump + relay + safety margin

6. Real-Time Operation

- Arduino works in real-time: checks moisture every second.

- Pump control uses hysteresis to avoid frequent on/off cycles.

- LCD shows live data; Serial Monitor is used for real-time monitoring.

- Project Source Code

- #include <LiquidCrystal_I2C.h>

// Инициализация дисплея
LiquidCrystal_I2C lcd(0x27, 16, 2);

// Пины подключения
const int sensorPin = A0; // Датчик влажности
const int relayPin = 7; // Реле

// Калибровочные значения (ПОМЕНЯЛИ МЕСТАМИ!)
int dryValue = 320; // Значение в СУХОЙ почве (маленькое число)
int wetValue = 620; // Значение во ВЛАЖНОЙ почве (большое число)

int humidityThreshold = 30; // Порог влажности в % (если < 30% - ВКЛЮЧИТЬ насос)

// Переменные для полива
bool isWatering = false;
unsigned long wateringStartTime = 0;
const unsigned long wateringDuration = 5000; // 5 секунд полива

void setup() {
Serial.begin(9600);

// Инициализация дисплея
lcd.init();
lcd.backlight();
lcd.setCursor(0, 0);
lcd.print("Auto Watering");
lcd.setCursor(0, 1);
lcd.print("System Ready!");
delay(2000);
lcd.clear();

// Настройка реле
pinMode(relayPin, OUTPUT);
digitalWrite(relayPin, HIGH); // Выключить реле

Serial.println("System started - FIXED VERSION");
}

void loop() {
// Чтение датчика
int sensorValue = analogRead(sensorPin);

// Конвертация в проценты (ПРАВИЛЬНАЯ логика)
// Чем МЕНЬШЕ значение датчика - тем СУШЕ
// Чем БОЛЬШЕ значение датчика - тем ВЛАЖНЕЕ
int humidityPercent = map(sensorValue, dryValue, wetValue, 0, 100);
humidityPercent = constrain(humidityPercent, 0, 100);

// Вывод на дисплей
lcd.setCursor(0, 0);
lcd.print("Moisture: ");
lcd.print(humidityPercent);
lcd.print("% ");

// Отладочная информация в Serial
Serial.print("Sensor: ");
Serial.print(sensorValue);
Serial.print(" | Humidity: ");
Serial.print(humidityPercent);
Serial.print("% | Status: ");

// Логика полива (ИСПРАВЛЕННАЯ)
if (isWatering) {
lcd.setCursor(0, 1);
lcd.print("WATERING... ");
Serial.println("WATERING");

// Проверяем время полива
if (millis() - wateringStartTime >= wateringDuration) {
stopWatering();
isWatering = false;
lcd.setCursor(0, 1);
lcd.print("Watering Done!");
delay(2000);
}
} else {
// ЕСЛИ влажность МЕНЬШЕ порога - ВКЛЮЧАЕМ полив
if (humidityPercent > humidityThreshold) {
lcd.setCursor(0, 1);
lcd.print("TOO DRY! PUMP ON");
Serial.println("TOO DRY - STARTING PUMP");
delay(1000);
startWatering();
isWatering = true;
wateringStartTime = millis();
} else {
lcd.setCursor(0, 1);
lcd.print("Soil OK :) ");
Serial.println("OK");
}
}

delay(1000);
}

void startWatering() {
digitalWrite(relayPin, LOW); // Включить реле
Serial.println("PUMP: ON");
}

void stopWatering() {
digitalWrite(relayPin, HIGH); // Выключить реле
Serial.println("PUMP: OFF");
}